Ground Survey Findings: Slag Dump Resistivity Line

One-page summary for site management — August 2026

| 222 Readings taken | 38 Ground stations | ~67 m Line length | 0 Bad/faulty readings |
| --- | --- | --- | --- |

## What we did

A resistivity survey passes a small electrical current through the ground and measures how easily it flows. Material that conducts electricity well (e.g. wet, metal-bearing, or saline material) shows up as low resistivity; drier or cleaner material shows up as high resistivity. We surveyed a 67-metre line across the slag dump using 38 ground stations, taking 222 measurements at increasing spacing to see both shallow and deeper ground conditions.

## What we found

- The data is clean and reliable — every reading passed our quality checks, with no missing, duplicated, or faulty measurements.

- Resistivity values ranged from about 4.5 to 32 ohm-metres, in line with what's expected for slag/waste fill material rather than natural bedrock.

- Resistivity consistently drops with depth along this line — this is a real, statistically confirmed pattern (not just a visual impression), suggesting the material becomes more conductive (e.g. wetter, or more metal-bearing) at depth.

- A handful of specific stretches along the line stand out as notably more — or less — conductive than the rest. These are the most efficient places to target if a follow-up borehole or ground-truthing dig is planned.

## What this means for next steps

Because the below-ground trend is consistent and statistically clear, we recommend treating the identified conductive zones as priority locations for direct sampling (borehole or test pit) to confirm what's driving the signal — e.g. moisture, leachate, or metal content. The full technical workbook (with all charts, the underlying data, and an interactive dashboard) is available for the engineering team; this page is a plain-language summary for planning and budget discussions.

## Confidence and limits

This survey shows where resistivity is unusually high or low — it does not by itself confirm what's causing those patterns. A resistivity survey is a screening tool, not a substitute for physical sampling. The numbers above come from a standard, widely used calculation (apparent resistivity); a full computer inversion (a more advanced modeling step) would sharpen the depth accuracy further and is recommended before any final decision.

Prepared from field data collected by BGR Hannover (Germany); analysis and this summary prepared for internal planning use.